P3 Controller GUI Manual

About the GUI:

The Controller GUI is a wxPython application designed to configure and control actuators based on Barrett’s P3 motor controller.

Installation:

Install the Peak PCAN-USB driver:

http://www.peak-system.com/fileadmin/media/files/usb.zip

Install these dependencies from their respective web sites:

* python 2.7.10

https://www.python.org/ftp/python/2.7.10/python-2.7.10.msi

* wxpython 3.0.2

http://downloads.sourceforge.net/wxpython/wxPython3.0-win32-3.0.2.0-py27.exe

* matplotlib 1.4.2/1.4.3

http://sourceforge.net/projects/matplotlib/files/matplotlib/matplotlib-1.4.3/windows/matplotlib-1.4.3.win32-py2.7.exe/download

* numpy 1.9.2

http://sourceforge.net/projects/numpy/files/NumPy/1.9.2/numpy-1.9.2-win32-superpack-python2.7.exe/download

* pywin32 219

http://sourceforge.net/projects/pywin32/files/pywin32/Build%20219/pywin32-219.win32-py2.7.exe/download

Install these additional dependencies from the command prompt:

* cd c:\python27\scripts

* pip install python-dateutil pytz pyparsing six

Launching:

Connect the actuator to power and CAN.

Double-click the “ControllerGUI.bat” file to launch the GUI. You should see the screen in Figure 1:

Basic Operation:

Figure 1

The GUI will automatically initialize the CAN device and enumerate the bus to make a list of available pucks. If there is only one available puck, it automatically selects that one as the “Active Puck”. Otherwise, you must choose an active puck from the list.

If the actuator was configured to home itself automatically upon startup, or if you do not wish to home the actuator, press the “Skip Homing” button. This will enable the rest of the GUI, as seen in Figure 2:

Figure 2

If you leave the Waveform Characteristics at their defaults then press “Create Waveform”, the program will generate a waveform called “wave_pos_sin” which is a sinusoidal position waveform with a frequency of 0.5 Hz, an amplitude of 1000 (scaled) counts, and a Y-offset (bias) of 2000 (scaled) counts.

The full position range of the actuator after homing can be commanded by giving values between 0 and 4095. Therefore, this position waveform will start near the center of the actuator’s range (2000), then it will apply a sinusoidal waveform with amplitude 1000 at a frequency of 0.5 Hz (a period of 2 seconds).

The data is always streamed to the actuator at 30 Hz.

Once the waveform has been generated, select it from the “Waveforms” list, then press the Play button. See Figure 3:

Figure 3

The actuator should start moving in a sinusoidal pattern.

By default, the position control bandwidth is set pretty high for the relatively slow update rate of 30 Hz. Therefore, the actuator tends to reach the commanded position before an updated position is sent by the GUI. This results in somewhat choppy motion. To smooth out the motion, select the “Control Bandwidth” tab, then lower the Position Bandwidth from 5 Hz to 1 Hz. This should smooth out the actuator’s motion. See Figure 4:

Figure 4

You can pause the motion by pressing the Pause button. You can adjust the Waveform Characteristics in the left-most panel, then press “Create Waveform” to add your new waveform to the “Waveforms” list. Double-click your new waveform to activate and play it.

Congratulations, you have completed the initial walk-though of the Controller GUI!